Infodraw PMRS-102 Ruggedize Media Relay System

Product Specification

Overview

PMRS-102 Ruggedize is a smart, small, handheld metal IP-65, wearable streaming media system from the field over cellular network. It allows to stream video/audio/location live and monitoring video, audio and location over map anywhere upon alarm/request.

Key Features

- Handheld, wearable small form factor

- Real embedded system (design for cellular network)

- Live video/audio/location streaming (low latency)

- Dual encoding (recording and streaming)

- Dual resolution for recording/streaming D1/CIF accordingly

- PTZ control using RS-485

- Built-in cellular module (need only SIM)

- Built-in memory for local recording (128MB)

- Micro SD slot for local recording (up to SDHC 32g)

- Optional Encryption (RSA 1024+AES)

- High performance DSP based CODEC (H.264)

- 1 Analog video input CIF/D1

- 1 video over IP

- 1 audio over IP   (2 ways)

- Built-in GPS receiver

- Deep Sleep mode and scheduler (months) using battery only

- Monitoring video/voice and location over map

- Tracking history over map

- Bit rate/Frame rate remote control

- Automatic bandwidth control

- Alarm push button

- Motion detection

- SMS send according to events

- Built-in lithium battery 10Ah (10 hours streaming)

- Very low power 0.5 Amp while streaming (5v)

- Built-in 5v output to camera

- Recording on remote monitoring

- Web streaming using flash client

- Permission capability for different access level

- Upgrade firmware on the fly

- Small form factor 6.5 X 12 X 4 cm

- Built-in LAN interface

- Support iPhone and Android  phone clients

Technical Specifications

| Video |  |  |
| --- | --- | --- |
| Video Input | 2.5 mm video/audio/power 5v plug | PAL/NTSC |
| Video compression | H.264 Dual encoding (streaming/recording) | CIF/D1 |
| Bit rate | From 10kbs to 2Mbs |  |
| Frame rate | Up to 25/30 |  |
| PTZ Control | Built-in RS-485 (T+, T-) |  |
| Audio |  |  |
| Audio Input | Built-in microphone or Line in |  |
| Audio output | Line out |  |
| Audio compression | AAC |  |
| GSM |  |  |
| GSM | Built-in (need only SIM) | HSPA/3G/EDGE/GPRS |
| Antenna | Built-in |  |
| Bandwidth control | Automatic |  |
| GPS |  |  |
| GPS | Built-in | Update every few sec |
| Antenna | External |  |
| Alarm Button |  |  |
| Push button | On surface |  |
| Memory |  |  |
| Micro SD | Slot (max bit rate according to micro SD write speed) | Up to 32g |
| I/O |  |  |
| 4 Input/4 output | Built-in Through cable interface | Dry contact |
| Power |  |  |
| Power Input | 5V | AC/DC to 5V adapters |
| Power consumption | 500mA while streaming |  |
| Battery Lithium | Built-in 10Ah (10 hours streaming) |  |
| Power output | 5v output to camera |  |
| Environment |  |  |
| Water proof | IP-65 |  |
| Temperature Input | -10 +70 |  |
| Physical |  |  |
| Dimension | 6.5 x 12 x 4 cm |  |
| Weight | 200 gr |  |
| Other Interfaces |  |  |
| USB client | Built-in | For Setup |
| video/audio/power cable | Splitter cable with RCA, audio, 5V power |  |
| Earphone/mic cable |  |  |
| LAN | 10/100 Built-in Through cable interface |  |
| Protocols |  |  |
| RTP/RTSP/SDP |  |  |

PMRS-102 System Advantages:

- Handheld, wearable small form factor

- MRS architecture is based on MRS server

- Units can stream with the highest bandwidth exist

regardless number of clients

- Dual encoding (local recording in D1 and streaming in CIF is possible)

- System design for low bandwidth and packet delivery failure

- Automatic bandwidth control by the units

- Multi clients to multi units communication using the server

- Only MRS server must have fix IP address

- Live video, voice and location over map are combine together in one monitoring application

- Built-in cellular module (need only SIM)

- Deep Sleep mode and scheduler (remote control)

Figure 1- MRS System Architecture

Figure 2- MRS Demo monitoring