Infodraw PMRS-102HD Personal Media Relay System

Product Specification

Overview

PMRS-102HD is a smart, small, handheld, wearable streaming media system from the field over cellular network. It allows to stream video/audio/location live and monitoring video, audio and location over map anywhere upon alarm/request.

Key Features

- 3G or 4G device

- Handheld, small form factor

- Real embedded system (design for cellular network)

- Live video/audio/location streaming (low latency)

- Dual encoding (recording and streaming) HD resolution

- Dual resolution for recording/streaming

- Built-in cellular module (need only SIM)

- Built-in memory for local recording (128MB)

- Micro SD slot for local recording (up to SDHC 32g)

- Optional Encryption (RSA 1024+AES)

- High performance DSP based CODEC (H.264)

- 1 HDMI video/audio input

- 1 video over IP

- 1 audio over IP   (2 ways)

- Built-in GPS receiver

- Monitoring video/voice and location over map

- Tracking history over map

- Bit rate/Frame rate remote control

- Automatic bandwidth control

- Alarm push button

- Motion detection

- SMS send according to events

- Built-in lithium battery (5 hours streaming)

- Very low power 0.5 Amp while streaming (5v)

- Recording on remote monitoring

- Web streaming using flash client

- Permission capability for different access level

- Upgrade firmware on the fly

- Small form factor 5 X 10 X 3 cm

- Built-in LAN interface

- Support iPhone and Android  phone clients

Technical Specifications

| Video |  |  |
| --- | --- | --- |
| Video Input | HDMI video/audio |  |
| Video compression | H.264 Dual encoding (streaming/recording) |  |
| Bit rate | From 10kbs to 2Mbs |  |
| Frame rate | Up to 25/30 |  |
| Audio |  |  |
| Audio Input | Built-in microphone or Line in |  |
| Audio output | Line out |  |
| Audio compression | AAC |  |
| GSM |  |  |
| GSM | Built-in (need only SIM) | 4G or 3G |
| Antenna | Built-in |  |
| Bandwidth control | Automatic |  |
| GPS |  |  |
| GPS | Built-in | Update every few sec |
| Antenna | External |  |
| Alarm Button |  |  |
| Push button | On surface |  |
| Memory |  |  |
| Micro SD | Slot (max bit rate according to micro SD write speed) | Up to 32g |
| I/O |  |  |
| 4 Input/4 output | Built-in Through cable interface | Dry contact |
| Power |  |  |
| Power Input | 5V-15V | AC/DC to 5V adapters |
| Power consumption | 500mA while streaming |  |
| Battery Lithium | Built-in 4000 mAh (5 hours streaming) |  |
| Environment |  |  |
| Temperature Input | -10 +60 |  |
| Physical |  |  |
| Dimension | 5 x 10 x 2.8 cm |  |
| Weight | 130 gr |  |
| Other Interfaces |  |  |
| USB client | Built-in | For Setup |
| video/audio/power cable |  |  |
| Earphone/mic cable |  |  |
| LAN | 10/100 Built-in Through cable interface |  |
| Protocols |  |  |
| RTP/RTSP/SDP |  |  |